Terminal operator DIALOG 640

Display

- LCD display with CFL backlight

- Display area 122 x 66 mm

- Character attributes : normal, reverse ,blinking

- ASCII protocol

- Resolution (pixels) in graphic mode : 240 x 128

- 4 simultaneous sizes of characters in text mode :

- 3x4 mm	16 lines x 40 characters

- 4x7 mm	9 lines x 30 characters

- 5x8 mm	8 lines x 26 characters

- 7x10 mm	6 lines x 17 characters

Keyboard

- 33 keys with tactile feedback

- 6 dynamic function keys

- 6 re-legendable function keys with integrated leds

- Control and scrolling keys

- Help key

- Alarm key

- Numeric and alphanumeric keypad

- Buzzer

Performances

- 16 bit processor

- 512 Kbyte FLASH memory

- 128 Kbyte non-volatile RAM

- RS232 serial communication port

- Optional RS422 or RS485 serial communication port

- Optional fieldbus : CANBUS

Technical features

- 24Vdc supply voltage

- 6W power requirement

- 0 to 45°C operating temperature

- -20 to 70°C storage temperature

- Front panel protection IP65

Software

- 4 languages

- 200 screens

- 50 objects maxi per screen

- Number of objects :

1000 fixed texts, vertical lines, horizontal lines ...

1000 dynamical texts

1000 numeric or alphanumeric variable display

1000 numeric or alphanumeric variable edition

- 4 fonts : normal or reverse

- Display and edition format :

decimal

hexadecimal

binary

ascii

- Help screens with led indicator

- Alarm screens with led indicator :

320 alarms maxi

Parameters on each alarm : acknowledge or not, help screen

- Global or local function keys

- Programmable functions keys :

Press function key

Flip / flop function key

- Function keys with led indicator :

on

off

blink

- Boxes integrated :

Confirmation box

Password box

- 10 programmable passwords

- 5000 variables :

word or dword type

numeric or alphanumeric format

- 10000 variables for program ( example : 200 programs of  50 elements )

- Communication array enable or not

- RS 232 or RS 485 Master Modbus RTU driver